# FICHE DE PROCÉDURE – Configuration MUTLAB (VLAN + SSH)

# Pré-requis

• Accès console au switch Cisco 2960 (MUTLAB).

• Câble réseau branché sur un port affecté au VLAN de management.

• Adressage validé selon le contexte GSB (passerelles .254 sur rtrout).

# Étapes

## 1) Configuration de base

conf t
no ip domain-lookup
service password-encryption
hostname MUTLAB
banner motd # Accès réservé – Connexions non autorisées interdites #
enable secret NetMSISR
line console 0
 login
 password Console@GSB
 exec-timeout 10 0
 logging synchronous
line vty 0 4
 login local
 transport input ssh
 exec-timeout 15 0
username admin secret NetMSISR

## 2) Création des VLANs demandés

vlan 1
 name VLAN par défaut
vlan 10
 name Réseau & Système
vlan 50
 name Développement
vlan 99
 name Admin (Management)
vlan 150
 name Visiteurs
vlan 200
 name Direction / DSI
vlan 201
 name RH / Compta / Juridique / SA
vlan 202
 name Communication / Rédaction
vlan 203
 name Commercial
vlan 204
 name Labo-Recherche
vlan 205
 name Accueil
vlan 206
 name Démonstration
vlan 250
 name Serveurs

## 3) Affectation des ports (mode access)

interface fa0/1
 switchport mode access
 switchport access vlan 1
 spanning-tree portfast
interface fa0/2
 switchport mode access
 switchport access vlan 1
 spanning-tree portfast
interface fa0/3
 switchport mode access
 switchport access vlan 10
 spanning-tree portfast
interface fa0/4
 switchport mode access
 switchport access vlan 10
 spanning-tree portfast
interface fa0/5
 switchport mode access
 switchport access vlan 50
 spanning-tree portfast
interface fa0/6
 switchport mode access
 switchport access vlan 50
 spanning-tree portfast
interface fa0/7
 switchport mode access
 switchport access vlan 99
 spanning-tree portfast
interface fa0/8
 switchport mode access
 switchport access vlan 99
 spanning-tree portfast
interface fa0/9
 switchport mode access
 switchport access vlan 150
 spanning-tree portfast
interface fa0/10
 switchport mode access
 switchport access vlan 150
 spanning-tree portfast
interface fa0/11
 switchport mode access
 switchport access vlan 200
 spanning-tree portfast
interface fa0/12
 switchport mode access
 switchport access vlan 201
 spanning-tree portfast
interface fa0/13
 switchport mode access
 switchport access vlan 202
 spanning-tree portfast
interface fa0/14
 switchport mode access
 switchport access vlan 203
 spanning-tree portfast
interface fa0/15
 switchport mode access
 switchport access vlan 204
 spanning-tree portfast
interface fa0/16
 switchport mode access
 switchport access vlan 205
 spanning-tree portfast
interface fa0/17
 switchport mode access
 switchport access vlan 206
 spanning-tree portfast
interface fa0/21
 switchport mode access
 switchport access vlan 250
 spanning-tree portfast
interface fa0/22
 switchport mode access
 switchport access vlan 250
 spanning-tree portfast

## 4) IP de management (SVI) + passerelle par défaut

interface vlan 250
 ip address 192.168.250.70 255.255.255.0
 no shutdown
ip default-gateway 192.168.250.254

## 5) Activer SSH (RSA 1024 bits mini)

ip domain-name gsb.local
crypto key generate rsa
 1024
ip ssh version 2

## 6) Sauvegarde

end
write memory

# Notes

• Les .254 sont les passerelles sur rtrout par convention GSB. Si votre salle utilise une autre IP de passerelle, adaptez la commande ip default-gateway.

• Le switch 2960 est un L2 : les SVI servent uniquement à l’administration (pas de routage inter-VLAN).